| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 12 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2011 |  | TD 337 (GEN/12) |
|  |  | English only Original: English |
| Question(s): | 14/12 |  |
| TEMPORARY DOCUMENT |  |  |
| Source: | Rapporteurs for Question 14/12 |  |
| Title: | Updated P.NBAMS Terms of Reference (ToR) |  |

# Abstract

The P.NBAMS work item in Q.14/12, for an objective bitstream video quality model, has ToR which are similar to the ToR for P.NAMS. This document contains the stable P.NBAMS ToR, updated after the May 2010 Study Group 12 meeting.

- Scope

The current scope of P.NBAMS (Non-intrusive bitstream model for the assessment of performance of video streaming) is given below:

This Recommendation describes an objective bitstream quality assessment model that predicts the impact of observed coding and IP network impairments on quality experienced by the end-user in multi-media mobile streaming and IPTV applications over transport formats such as RTP (over UDP), MPEG2-TS (over UDP or RTP/UDP), and 3GPP-PSS (over RTP).

This model can use the bitstream information in addition to information contained in packet headers, prior knowledge about the media stream, and buffering information from the client.

The model has two modes. In the first mode the model does not completely decode the payload. This means that no pixel information is used internally in the model. In the second mode the model decodes parts or all of the video sequence and the pixel information is used for the MOS estimation, in addition to other bitstream information.

The first version of this model predicts Mean Opinion Scores (MOS) on the five point ACR scale for the video part of the stream. The audio and multi-media MOS score may be considered in a second phase of P.NBAMS (Using the audio and multimedia part of P.NAMS is another option).

The primary applications for this model are monitoring of transmission quality for operations and maintenance purposes. The P.NBAMS model may be deployed both in end-point locations and at mid-network monitoring points.

The primary quality prediction made by such a model is based on the payload of the stream being analyzed. Therefore, this Recommendation can provide a comprehensive evaluation of quality as perceived by a particular end-user because its scores can reflect the impairments on the coding and the IP network being measured, which differ from user to user.

- General Application areas

The two general application areas for P.NBAMS are:

- QCIF-QVGA (also known as “low bitrate mode”), mostly for mobile TV and streaming with the following sub-application areas.

  - Linear Mobile TV over RTP (includes Mobile TV over a 3G mobile network with MBMS and with unicast and transport over RTP/UDP/IP)

  - Multimedia streaming, progressive download, and download (includes progressive download techniques over TCP/IP and 3GPP PSS with transport over RTP/UDP/IP)

- SD and HD television (also known as “high bitrate mode”), mostly for IPTV with the following sub-application areas.

  - Broadcast TV (includes transmission over MPEG2-TS/RTP/UDP/IP, MPEG2-TS/UDP/IP transport).

  - Video on-demand (includes transmission over MPEG2-TS/RTP/UDP/IP, MPEG2-TS/UDP/IP, and RTP/UDP/IP transport)

- Modes

  - Decoding modes

The P.NBAMS model has two decoding modes:

- Mode 1: Parsing mode – in this mode the model does not completely decode the payload. This means that no pixel information is used internally in the model. Any kind of analysis of the bitstream, without using pixel information, can be done. 

Note: The computational complexity and resource requirements of a mode 1 model makes it suitable for implementation in a end-point device (for example set-top box) or network element.

- Mode 2: Full decoding mode – in this mode the model decodes parts or all of the video sequence, and the pixel information is used for the MOS estimation, in addition to other bitstream information. 

Note: The computational complexity and resource requirements of a mode 2 model makes it suitable for implementation in measurement equipment.

  - Operation modes

The four modes of operations are described in Table 1 and Figures 1 a) to e) below.

| Name | Mode abbreviation* | Description |
| --- | --- | --- |
| Static Operation | NN | The model uses information from the local transport layer and payload, prior knowledge about coding and prior knowledge about the end-point. |
| Non-embedded Dynamic Operation | BN | The model uses information from the local transport layer and payload, prior knowledge about coding and information about the end-point collected through measurement reporting protocols. Example of end-point information is buffer behaviour and type of error concealment. |
| Non-embedded Distributed Operation | CN | The model, located inside the network, uses information from the transport layer measured at an end-point and collected through signalling protocols, prior knowledge about coding and information about the end-point collected through signalling protocols. The extracted parameters can also be used internally in a different module of the end-point device. |
| Embedded Operation | CC | The model uses information from the local transport layer, information from the end-point, and prior knowledge about coding. Example of end-point information is buffer behaviour and type of error concealment. |

Table 1: Modes of operations of P.NBAMS.

*Mode abbreviation naming scheme:

XY, where

X corresponds to place of measurement (N: Network, C: Client, B: Both network and client)

Y corresponds to place of model (N: Network, C: Client)

Figure 1 a): Static operation mode (NN) inside the network

Figure 1 b): Static operation mode (NN) inside a terminal

Figure 1 d): Non-embedded dynamic operation mode (BN)

Figure 1 e): Non-embedded distributed operation mode (CN)

Figure 1 f): Embedded operation mode (CC)

- Model Input

The P.NBAMS consists of one building block, and how this block is combined with blocks from P.NAMS is described in Figure 2 below.

Figure 2: P.NBAMS Building Block and combination with P.NAMS modules

Figure 3 shows how the case of error resilience methods such as Forward Error Correction (FEC) and Automatic Repeat reQuest (ARQ) shall be handled in case of the NN mode; here, due to the measurement in the network the stream has not been corrected by the resilience methods. See Section 5 for more details.

Figure 3 a): Options for handling of FEC/ARQ in P.NBAMS, parameter extraction module (P-E). Note that the “Loss-related parameter mapping” module is considered transparent for all non loss-related parameters.

Edge device (e.g., STB)

FEC

decoding

Demultiplex

ing

P.NBAMS

P.NAMS

Header

information

Payload bitstream

information

IP stream

Baseband Video and audio

DRM

decryption

Parser

Decoder

Video quality

Audio quality

Multimedia quality

PLC

information

buffer

info

Figure 3 b): Embedded operation mode (CC) including FEC decoding

Figure 3 c): Static operation mode (NN) using independent monitoring devices
Editors note: This scenario might be applicable for the other NN variants and also for BN mode. See figure 3 below.

- Model Input

The P.NBAMS model will receive transport layer information, bitstream (payload) information, client information, and prior knowledge about the media stream. The overview information per application area and mode is described in Table 2a below:

|  | Static Operation (NN) | Non-embedded Dynamic Operation (BN) | Non-embedded Distributed Operation (CN) | Embedded Operation (CC) |
| --- | --- | --- | --- | --- |
| Mobile application area - Low bitrate mode | Prior knowledge about video coding, and end-point as described in Table 2.b PCAP file including bitstream | Prior knowledge about video coding, and end-point as described in Table 2.b PCAP file including bitstream Additional endpoint information provided via signaling protocol. Note: Protocol not known and defined | Prior knowledge about video coding, and end-point as described in Table 2.b Parameters extracted from bitstream in client provided via signaling protocol. Note: Protocol not known and defined | Prior knowledge about video coding, and end-point as described in Table 2.b PCAP file including bitstream Freezing information (type, start and duration of freezing) |
| High bit-rate mode | Prior knowledge about video coding, and end-point as described in Table 2.b PCAP file including bitstream | Prior knowledge about video coding, and end-point as described in Table 2.b PCAP file including bitstream Additional endpoint information provided via signaling protocol Note: Protocol not known and defined | Prior knowledge about video coding, and end-point as described in Table 2.b Parameters extracted from bitstream in client provided via signaling protocol Note: Protocol not known and defined | Prior knowledge about video coding, and end-point as described in Table 2.b PCAP file including bitstream Freezing information (start and duration of freezing) |

Table 2a: Overview of input to the P.NBAMS application areas and modes of operation

The following Table 2b gives detailed examples of prior knowledge that is provided at to P.NBAMS.

| Modes | Parameter | Description | Notes |
| --- | --- | --- | --- |
| All | Video decoder | Video decoder and settings used | Optional. If not available assume typical decoder |
| NN | Client buffer size | Size of client buffer | bytes/ms? Not needed when buffering information is received from decoder If not available assume typical buffer size |
| All | Client buffer and packet loss behaviour | - Freezing or display slicing errors when packets are lost - If freezing: with or without skipping (freezing without skipping only for VoD services) | 2. Freezing alternatives: Only valid for mobile application area. Should be depending only on service, not on client |
| CN | Payload-unit-start indicator | Information on whether payload-unit-start indicator in TS header means the start of frame or of slice | Not needed for P.NBAMS? |
| CN | Random-access indicator | Information on whether random-access indicator in TS header means the start of I-frame or of IDR-frame | Not needed for P.NBAMS? |
| CN | Elementary-stream priority indicator | Information on whether elementary-stream-priority indicator in TS header means the start of I-frame or of IDR-frame | Not needed for P.NBAMS? |
| CN | Marker bit | Information on whether marker bit in RTP header means the start of frame or of slice | Not needed for P.NBAMS? |

Table 2b: Examples of information available to the P.NBAMS model, prior knowledge.

Note that for the modes CC, CN and BN and UDP-based transport, it is assumed that the available information already reflects the impact of the employed Forward Error Correction (FEC), packet re-transmission mechanisms such as Automatic Repeat reQuest (ARQ), and the de-jitter buffer, since the parameter extraction module is located behind these processing steps.

In case of the NN mode, the measurement point is located prior to the actual FEC, ARQ and de-jitter buffer mechanisms. Since these mechanisms may have a very strong impact on factors such as the packet loss seen by the decoder, the NN-mode requires an explicit handling of these mechanisms. Two principle approaches are conceivable to capture this case, which are both based on prior knowledge:

- The packet stream is converted into a stream that reflects an assumed behaviour of FEC, ARQ, and/or jitter de-buffering, reflecting the input format to be provided to the parameter extraction module (PCAP). This option is not shown in Figure 2.

- The input parameters to the model are converted into values that reflect the application of FEC, ARQ and/or de-jitter buffering (module 1.C in Figure 2).

For the basic operation of the model described in this ToR, option b) is chosen.

As a first step, the packet loss rate is converted into the packet loss rate to be expected after FEC, ARQ and/or de-jitter-buffering. A corresponding module implementation is planned to be developed jointly with Q.17/12. An optional conversion of other packet-loss distribution related parameters is conceivable and will be considered further, also collaborating with Q.17/12.

There are two implementation alternatives for P.NBAMS for option b) above:

- Use P.NBAMS for estimating the quality without any packet loss. The impact of packet loss is estimated using similar technique as in P.NAMS. The normal P.NBAMS packet loss estimation is not valid for this case.

- Use P.NAMS instead of P.NBAMS for the NN mode in combination with FEC/ARQ.

Note that FEC and ARQ are not relevant for mobile Video services, and hence are not considered in the LBR-case.

The TCP-inherent ARQ-like mechanism of retransmission is thought to be conveniently captured at a well chosen measurement point, regardless of the mode of operation. Consequently, in case of TCP-based transport, it is assumed that all retransmission-related information is available to the model, and no explicit handling of ARQ like in the UDP-based transport is required (see above explanations for UDP). This, in turn, requires that for TCP-based transport the parameter extraction module considers all available packets including the retransmitted ones.

- Model Output

The P.NBAMS model output video MOS. P.NBAMS will have four modes, providing different MOS outputs for the two video resolutions for the two different application areas, see Sections 2 and 3.

Optional parameters for diagnostic purposes can be found in the (non-exhaustive) list below:

- Jitter buffer size

- Bit rate (audio and video)

- Packet-loss ratio (audio and video)

- Packet-discard ratio (audio and video)

- Averaged burst packet loss length (audio and video)

- Burst packet loss variation range

- Mean packet size

- Delay

- Delay variation range

- Application Range

In the following, the application ranges for the two application areas are defined in more detail (see Table 3 for the mobile, and Table 4 for the IPTV-case).

The four last columns of Table 3 and 4 give an indication on whether the respective information is considered as available for the respective operation mode. In case that the application information is not or only partly provided, corresponding additional specifications are required.

  - Application Range for Mobile Application Area

[Editor’s note : All aspects shown with yellow highlighting will be agreed on during further P.NBAMS model development.]

| Application information | Value range, unit | Comments |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Mode NN | Mode BN | Mode CN | Mode CC |
| Sequence duration (Ts) | 10-24 s (maximum 33% of the sequence length is rebuffering, and maximum source sequence length 16 s) |  |  |  |  |
| Packetization | 3GPP MBMS, PSS or using RTSP directly (all three over RTP/UDP/IP) HTTP/TCP/IP, or [RTP/]TCP/IP |  |  |  |  |
| Video codec | H.264 baseline profile |  |  |  |  |
| Video size | QCIF, QVGA, HVGA |  |  |  |  |
| Coded video bitrate | QCIF: default 80 kbps, 32 - 256 kbps QVGA: default 250 kbps, 80 - 800 kbps HVGA: default 800 kbps, 150 - 2 Mbps |  |  |  |  |
| Decoder packet loss concealment | Two types of assumed decoder behaviour: 1) Freezing with skipping, 2) slicing with N slices/frame, PLC to be agreed (e.g. fixed decoder) |  |  |  |  |
| Client buffer behaviour | Rebuffering handling, particular to LBR-case; freezing with and without skipping of length 0 to Y seconds (Y 8 s, depends on source sequence length) | Client buffer emulation (PCAP-file conversion or parameter conversion) Tbd to be discussed with Q.17/12 | Depends on reporting protocol Tbd. | Depends on reporting protocol. Tbd. | Measure-ment after client buffering available |
| Encoder implementation | No limitation needed. Model should handle different encoders (should be possible to decode with reference decoder) |  |  |  |  |
| Reference decoder for bitstream verification | Reference decoder: JM decoder for H.264. |  |  |  |  |
| Decoder implementation for subjective testing | To be further discussed. Could be specified in the test plan. Reference decoder: JM decoder for H.264. Possibly error concealment type as model input. If so, must be further defined. Guidance on how to adjust internal model parameters for specific other decoders to be added as Appendix of Recommendation or stand-alone Rec. (see decoder PLC). |  |  |  |  |
| Group of pictures (GoP) | Assume typical GoP Structure for mobile case: No B-frames (M=1). M = 1, N in range 10 – 300 or even only one I-frame in sequence (typical M=1, N=50) Length: fixed, variable |  |  |  |  |
| Slice structure | Allowed structures: - One slice per row - Slice based on packet size - No slicing |  |  |  |  |
| Frame rate | 5, 6.25, 7.5, 8.33, 10, 12.5, 15, 24, 25, 30 fps |  |  |  |  |
| Type of Video Content | Typical mobile TV and streaming content |  |  |  |  |
| Packet loss degradation | Uniform loss: 0-10% Burst loss: 0-10%, values to be agreed as well as additional parameters (collaboration with Q.17/12, examples are loss and gap run-lengths, etc., e.g. based on N-state Markov model) | FEC and ARQ assumed to not play a role for mobile application area. |  |  |  |

Table 3: Overview of application range of P.NBAMS for the mobile application area.

  - Application Range for IPTV Application Area

[Editor’s note : All aspects shown with yellow highlighting will be agreed on during further P.NBAMS model development]

| Application information | Value range, unit | Comments |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Mode NN | Mode BN | Mode CN | Mode CC |
| Sequence duration (Ts) | Source length: 10s PVS length = SRC length |  |  |  |  |
| Packetization | MPEG2-TS/RTP/UDP/IP RTP/UDP/IP MPEG2-TS/UDP/IP HTTP/TCP/IP MPEG2-TS/TCP/IP [RTP/]TCP/IP |  |  |  |  |
| Video codec | MPEG2, H.264 baseline profile, H.264 main profile, H.264 high profile, AVS? |  |  |  |  |
| Video size and frame rate | SD: PAL with frame rates 24p, 25p , 50i NTSC with frame rates 24p, 30p, 60i HD: 720p with frame rates 24p, 25p, 30p, 1080p with frame rates 24p, 25p, 30p 1080i with frame rates 50i, 60i Note: The frame rates 29,97/30 and 59.94/60 are considered equivalent |  |  |  |  |
| Coded video bitrate | HD H.264: default 9 Mbps, 3 - 15 Mbps SD MPEG2: default 4 Mbps, 3 - 10Mbps H.264: default 2.1 Mbps, 1.5 up to 6 Mbps |  |  |  |  |
| Video decoder packet loss concealment | Types of assumed decoder behavior: 1) Freezing with skipping (T(SRC)=T(PVS)), 2) slicing with 1 slice/frame, no PLC. 3) slicing with N slices/frame, PLC to be agreed (e.g. fixed decoder) |  |  |  |  |
| Retransmission mechanisms (ARQ); Forward Error Correction (FEC) | Type and range tbd. | To be discussed. See section 5 above. | Measure-ment after ARQ, FEC available, depends on description protocols in same way as for parameters when no ARQ is used | Measure-ment after ARQ, FEC available, depends on description protocols in same way as for parameters when no ARQ is used | Measure-ment after ARQ, FEC available |
| Client jitter buffer behaviour | tbd. (information to the right taken from P.NAMS ToR) | Jitter buffer emulation (PCAP-file conversion or parameter conversion, see Figure 3a) Tbd with Q.17/12 1) Required: Loss-rate -conversion 2) optional burst-related measure 3) optional: additional parameters | Depends on reporting protocol. tbd | Depends on reporting protocol. tbd | Measure-ment after de-jitter buffering available (detailed information about freezing/rebuffering available) |
| Type of Video Content | Typical for an IPTV service |  |  |  |  |
| Encoder implementation | No limitation needed. Model should handle different encoders |  |  |  |  |
| Reference decoder implementation (to verify bitstream) | H.264: JM decoder? TBD MPEG2: TBD. Can be aligned with VQEG hybrid project |  |  |  |  |
| Decoder implementation | To be further discussed. Can be moved to the test plan H.264: JM decoder? TBD MPEG2: TBD Guidance on how to adjust internal model parameters for specific other decoders incl. set-top boxes to be added as Appendix of Recommendation or stand-alone Rec. (see decoder PLC). |  |  |  |  |
| Post processing after decoding (should be moved to the test plan) | De-interlacing: One typical de-interlacing algorithm TBD De-blocking filtering MPEG2: One typical, or none: TBD Note. De-blocking filtering valid only for MPEG2, not H.264. De-blocking filtering is included in decoding standard for H.264) |  |  |  |  |
| Group of pictures (GoP) | Typical GoP structure for IPTV, including hierarchical B-frames. e.g., M = 3, N = 24 or M=3, N=15 Length: fixed, variable, adaptive Structure (e.g.: IBBPBB...PBBI) |  |  |  |  |
| Encrypted payload | No encryption |  |  |  |  |
| Slice structure | Allowed alternatives: - One slice per row - Slice based on packet size - No slicing |  |  |  |  |
| Packet loss degradation, video | Uniform loss: 0-4% (provisionally agreed; model will mainly be used for well-managed or partially managed networks (see G.1050), i.e. in packet loss range 0-2%) Burst loss: 0-2%, values to be agreed as well as additional parameters (collaboration with Q.17/12, examples are loss and gap run-lengths, etc., e.g. based on N-state Markov model) |  |  |  |  |

Table 4: Overview of application range of P.NBAMS for the IPTV application area.

- Requirements

Should a requirement/objectives table be included here or not?

- References

[1]	3GPP TS 26.234 “Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs”

[2]	3GPP TS 26.364 “Multimedia Broadcast/Multicast Service (MBMS); 
Protocols and codecs “

[3]	Broadband Forum Technical Report. TR-135. “Data Model for a TR-069 Enabled STB”. Version: Issue 1. December 2007.

[4]	IETF RFC 3611 “RTP Control Protocol Extended Reports (RTCP XR)”, November 2003.

[5]	Development/LibcapFileFormat, http://wiki.wireshark.org/Development/LibpcapFileFormat.

______________